PulseCheck

Heart-rate estimation and deepfake screening from the pulse in ordinary face video

Technical report: design, implementation and evaluation

Version 0.1.0  ·  generated 11 October 2026

Results in this edition: heart-rate validation on UBFC-rPPG: not run yet; classical detector on FaceForensics++: not run yet; CNN detector on FaceForensics++: not run yet; compression study: not run yet. Every result is computed from the real datasets by the commands listed in Chapter 11; the report is regenerated with python -m pulsecheck report-doc.

# Contents

Table of contents: in Word, right-click here and choose Update Field.

# 1. Summary

PulseCheck measures a person's heart rate from an ordinary video of their face, without touching them, and uses the same pulse signal to screen videos for deepfakes. Every heartbeat pushes blood into the skin; blood absorbs green light, so the skin reflects slightly less green for a moment. A camera records this change. Measuring a pulse with light is photoplethysmography (PPG); doing it from video at a distance is remote PPG (rPPG).

The deepfake idea follows FakeCatcher (Ciftci, Demir and Yin, 2020): on a real face every patch of skin is fed by the same heart, so the patches' pulses rise and fall together. A face synthesised or swapped by a generator has no heart of its own, so its pulse tends to be missing, smeared, or out of step with the skin the generator did not touch.

The project has four parts:

A signal-processing pipeline (Python): face landmarks, skin patches, three classic rPPG algorithms (Green, CHROM, POS), filtering, Fourier analysis and heart-rate estimation.

Two detectors, both trained locally: classical scikit-learn models (CPU) on 91 hand-designed patch-agreement features, and a convolutional neural network (PyTorch, designed to train on the laptop's NVIDIA RTX 3050 GPU) on FakeCatcher-style PPG maps. Both use the same splits and are compared head to head.

An evaluation protocol designed to avoid flattering numbers: official subject-disjoint splits, grouped cross-validation, thresholds frozen on validation data, cross-dataset tests and a false-alarm test on genuine videos.

A web application (FastAPI + a static site with interactive Plotly charts), packaged with Docker for deployment on Hugging Face Spaces, where anyone can upload or record a clip and see every step of the analysis.

Results pending. Chapters 7 to 9 are filled in automatically from the real datasets once the commands in Chapter 11 have been run on them.

# 2. Background

## 2.1 Why skin colour carries a pulse

Light entering the skin is partly absorbed by haemoglobin. With each heartbeat the volume of blood in the microvessels rises and falls, so the amount of light absorbed rises and falls too. Haemoglobin absorbs green light most strongly, which is why the green channel of a camera carries the clearest pulse (Verkruysse et al., 2008). The change is tiny: typically well under one percent of the pixel value, i.e. a fraction of one grey level. It is recoverable only because thousands of pixels are averaged, which cancels sensor noise.

## 2.2 Why this can expose a deepfake

A deepfake generator is trained to make faces look right frame by frame; it is not trained to reproduce the sub-grey-level, spatially coherent colour rhythm of blood flow. FakeCatcher used this: it extracted pulse signals from 32 face regions, arranged them into 'PPG maps', and trained a CNN to tell real from fake, reporting about 96% accuracy on an older benchmark and about 91% on its own harder set.

Important caveat. Seibold et al. (2025, Frontiers in Imaging) showed that high-quality face swaps can inherit the pulse of the source video they were made from. A believable pulse is therefore evidence, not proof, and this project presents its output as a screening signal.

# 3. System overview

Figure 1. The pipeline. The top row is shared; the classical and the CNN detector branch off the colour traces.

| Stage | Module | What it does |
| --- | --- | --- |
| 1 Read video | pulsecheck/video.py | Decodes frames with OpenCV and recovers a trustworthy timestamp for each frame. |
| 2 Find the face | pulsecheck/face.py | MediaPipe Face Landmarker places 478 3-D points on the face in every frame. |
| 3 Skin patches | pulsecheck/roi.py | Six named regions plus a 32-patch grid; a per-video skin-colour model removes hair and brows. |
| 4 Traces | pulsecheck/pipeline.py, traces.py | Mean R, G, B of every patch per frame, resampled to a uniform 30 Hz clock and cached. |
| 5 Pulse | pulsecheck/rppg.py | Green, CHROM and POS turn three colour traces into one pulse waveform. |
| 6 Heart rate | pulsecheck/signals.py | Detrend, band-pass 0.7-3 Hz, Fourier transform, peak, SNR. |
| 7a Features | pulsecheck/features.py | 91 numbers per 8 s window describing how well the patches agree. |
| 7b PPG maps | pulsecheck/deep.py | 2 x 32 x 224 images of the grid patches' pulses for the CNN. |
| 8 Decide | detector.py, deep.py | Each window gets P(fake); the video score is their mean; a threshold gives the verdict. |
| 9 Evaluate | hr_eval.py, detect_eval.py, studies.py | Protocols that write reports/*.json for the website and this report. |

Table 1. Stages and the code that implements them.

# 4. Implementation

## 4.1 Reading video and keeping time

Every beats-a-minute number depends on knowing when each frame was captured. Files from phones and from browsers (WebM recorded with MediaRecorder) are often variable-frame-rate and report a meaningless nominal frame rate, so the reader collects both the decoder's presentation timestamp and frame-index/FPS for every frame, uses the presentation timestamps when they are complete and strictly increasing, and otherwise falls back to the index clock. All traces are then resampled by linear interpolation onto a uniform 30 Hz grid, so the rest of the code never deals with irregular timing. Frames larger than 720 px on the long side are shrunk with area interpolation first; averaging a patch is almost unaffected by this, and landmarking is faster.

## 4.2 Face landmarks

MediaPipe Face Landmarker (Tasks API, model face_landmarker.task, float16, SHA-256 verified on download) returns 468 face-mesh points plus 10 iris points. It runs in VIDEO mode, which tracks the face between frames instead of re-detecting it every frame (faster and steadier). Frames without a face are marked missing and filled by interpolation; a clip is rejected if a face is found in fewer than 60% of frames.

## 4.3 Skin patches and the skin-colour model

Two sets of patches are measured in every frame:

Six named regions for heart rate and the hand-made features: left and right forehead, left and right cheek, nose bridge and chin. Each is a polygon through fixed landmark numbers, chosen by plotting all landmark numbers on a face (python -m pulsecheck landmarks) and shrunk 8-12% towards its centre so its edges stay off brows and hairline.

A 32-patch grid for the CNN: small squares (side = 0.14 x the outer-eye-corner distance) centred on 32 landmarks across forehead, both cheeks, nose bridge and chin, ordered so neighbouring rows are neighbouring skin.

| Region | Landmark polygon |
| --- | --- |
| forehead left | 10, 338, 297, 333, 334, 296, 336, 9, 151 |
| forehead right | 10, 151, 9, 107, 66, 105, 104, 67, 109 |
| cheek left | 346, 347, 330, 266, 425, 411, 352 |
| cheek right | 117, 118, 101, 36, 205, 187, 123 |
| nose | 168, 351, 419, 248, 281, 5, 51, 3, 196, 122 |
| chin | 18, 313, 418, 262, 428, 199, 208, 32, 194, 83 |

Table 2. The six named regions (left/right are the subject's own).

Inside each patch, hair, eyebrows, beard and glasses frames must not be averaged with skin. Fixed skin-colour thresholds are known to work worse on darker skin, so instead PulseCheck learns a skin model from each video: it fits a 2-D Gaussian to the chrominance (Cr, Cb in YCrCb space, ignoring brightness) of the cheek pixels in the first ten frames, after trimming the 5% most extreme samples, and then keeps a pixel only if its squared Mahalanobis distance is below 13.8 (the 99.9% quantile of a chi-squared distribution with 2 degrees of freedom). If a mask would leave less than a quarter of a patch, the whole patch is used rather than failing.

## 4.4 From colour to pulse: Green, CHROM and POS

Each patch gives three numbers per frame: mean red, green and blue, written C(t) = [R, G, B]. Lighting changes and motion change all three channels together and are usually far larger than the pulse; the algorithms differ in how they cancel them. All outputs are oriented so that the waveform goes up when blood volume goes up.

| Method | Idea | Computation |
| --- | --- | --- |
| Green (Verkruysse 2008) | Use the channel blood affects most. | p(t) = -(G(t)/mean(G) - 1) |
| CHROM (de Haan & Jeanne 2013) | Two colour-difference signals in which brightness cancels, mixed with a tuned weight. | Cn = C/mean(C); X = 3Rn - 2Gn; Y = 1.5Rn + Gn - 1.5Bn; band-pass both; S = Xf - (sd(Xf)/sd(Yf)) * Yf; 1.6 s Hann windows, overlap-added |
| POS (Wang et al. 2017) | Project colour onto a plane orthogonal to the skin tone, then combine the two axes. | Cn = C/mean(C) per 1.6 s window; S1 = Gn - Bn; S2 = -2Rn + Gn + Bn; h = S1 + (sd(S1)/sd(S2)) * S2; overlap-add |

Table 3. The three rPPG algorithms implemented in pulsecheck/rppg.py.

The 1.6 s window is about one heartbeat long, so the weights can follow slow changes in lighting. The unit tests include a case where a lamp flickers at 1.8 Hz with twice the strength of the pulse: POS and CHROM still find the true rate while Green locks onto the flicker, which is exactly the reason those methods exist.

## 4.5 Filtering and reading the beats

Detrend: subtract the best straight line (scipy.signal.detrend) to remove slow drift.

Band-pass: zero-phase 3rd-order Butterworth filter (second-order sections, sosfiltfilt) keeping 0.7-3.0 Hz, i.e. 42-180 beats a minute. Zero-phase means the pulse is not shifted in time.

Spectrum: Hann-windowed periodogram, zero-padded 8x. A clip of length T only resolves frequencies 1/T apart (a 10 s clip at 30 fps is 300 numbers with steps 0.1 Hz = 6 bpm apart; 72 bpm = 1.2 Hz sits on step 12). Zero-padding adds no information, but interpolates the spectrum so a single dominant peak can be located between steps; the peak is refined further with a three-point parabola.

Heart rate = 60 x the strongest frequency inside the band.

Signal quality (SNR) after de Haan & Jeanne (2013): power within +/-0.1 Hz of the heart rate and +/-0.2 Hz of its first harmonic, divided by all other power between 0.5 and 4 Hz, in decibels. 0 dB means as much power off the pulse as on it.

## 4.6 Patch-agreement features (classical detector)

Each video is cut into 8 s windows with a 4 s hop. Pulses are computed once per patch over the whole clip (so filters have no edge effects inside a window) and then sliced. Each window becomes a row of 91 numbers:

| Family | Count | Definition |
| --- | --- | --- |
| Global | 10 | pooled SNR and spectral entropy; spread and range of the six per-patch heart rates; mean and minimum pairwise coherence and correlation; mean absolute lag; log ratio of 4-12 Hz to heart-band power (FakeCatcher's wide band) |
| Per patch (x6) | 36 | SNR at the pooled heart rate; gap between the patch's own rate and the pooled rate; correlation with the mean of the other five patches; high-band ratio; log amplitude relative to the median patch; log std of 4-12 Hz green flicker |
| Per pair (x15) | 45 | magnitude-squared coherence in the heart band, weighted by sqrt(Pxx*Pyy); Pearson correlation; lag of the best cross-correlation within +/-0.33 s |

Table 4. Feature families (full list in Appendix A).

Coherence deserves a note. Averaging coherence evenly across the 0.7-3 Hz band is dominated by the many frequencies that contain only noise and gave values around 0.3 even for patches that visibly share a beat. Weighting each frequency by the geometric mean of the two patches' own power concentrates the average where both patches carry energy, which on a real face is the heart rate, without favouring frequencies merely because the patches agree there.

## 4.7 Classical classifiers

| Model | Configuration |
| --- | --- |
| Logistic regression | median imputation, standardisation, L2, C = 0.3, class_weight = balanced |
| SVM (RBF) | median imputation, standardisation, C = 1, gamma = scale, Platt-scaled probabilities, balanced |
| Gradient boosting | HistGradientBoosting, 300 iterations, learning rate 0.05, 15 leaves, L2 = 1, balanced |
| Random forest | 400 trees, min 3 samples per leaf, balanced_subsample class weights |

Table 5. Candidate models (scikit-learn); the best by grouped cross-validation is kept.

## 4.8 CNN on PPG maps (trained on the GPU)

For each window the 32 grid patches' POS pulses form an image: 32 rows (patches) by 240 columns (samples). Channel 0 is the heart band (0.7-3 Hz) and channel 1 the wide band (0.7-12 Hz); every row is standardised. Training uses a random 224-sample crop of each window (a time-shift augmentation) and random sign flips (the polarity of a pulse says nothing about fakeness); evaluation uses the centre crop.

| Layer | Output shape | Parameters |
| --- | --- | --- |
| Input | 2 x 32 x 224 | - |
| Conv 3x9 -> BatchNorm -> ReLU -> MaxPool 1x2 | 16 x 32 x 112 | 16 x 2 x 27 + 32 |
| Conv 3x7 -> BatchNorm -> ReLU -> MaxPool 2x2 | 32 x 16 x 56 | 32 x 16 x 21 + 64 |
| Conv 3x5 -> BatchNorm -> ReLU -> MaxPool 2x2 | 64 x 8 x 28 | 64 x 32 x 15 + 128 |
| Conv 3x3 -> BatchNorm -> ReLU | 64 x 8 x 28 | 64 x 64 x 9 + 128 |
| Global average pool -> Dropout 0.3 -> Linear | 1 logit | 65 |

Table 6. CNN architecture (about 80,000 parameters).

Training: AdamW (learning rate 1e-3, weight decay 1e-2) with a cosine schedule, batch 256, binary cross-entropy with a positive-class weight equal to the real/fake ratio (FaceForensics++ has five fakes per real video), automatic mixed precision (float16) on CUDA, and early stopping when the validation video-level AUC has not improved for 8 epochs (maximum 40). The weights with the best validation AUC are kept. A model this size fits easily in the RTX 3050's 4 GB; the training windows stay in system memory as float16 and are moved to the GPU batch by batch.

## 4.9 From windows to a verdict

Each window's probability of being fake is averaged over the video (every window votes). The decision threshold is the one that maximises balanced accuracy on the validation split and is then frozen; the website says 'uncertain' when the score is within 0.1 of it. The website serves whichever detector, classical or CNN, has the higher validation AUC; the test set is never used to choose.

# 5. Datasets

| Dataset | Contents | Use here | Access and terms |
| --- | --- | --- | --- |
| UBFC-rPPG, DATASET_2 (Bobbia et al., 2019) | 42 uncompressed 640x480 30 fps webcam videos of people doing a maths game, with a finger pulse-oximeter PPG recorded alongside | heart-rate error; false alarms on genuine videos | public Drive folder; Agreement.xlsx; facial images of subjects 21 and 27 must not be published |
| FaceForensics++ c23 (Rossler et al., 2019) | 1,000 YouTube videos and 5 manipulations of each: Deepfakes, Face2Face, FaceSwap, NeuralTextures, FaceShifter | train / validate / test the detectors | request form; terms of use; no redistribution |
| Celeb-DF v2 (Li et al., 2020) | 590 celebrity + 300 YouTube real videos, 5,639 high-quality fakes; official 518-video test list | cross-dataset test only | request form; terms of use; no redistribution |

Table 7. Datasets. None is included in the repository or the deployed site.

FaceForensics++ ships official train/validation/test lists of source-target pairs (720/140/140 videos). Both videos of a pair and all their fakes share a group, so no person appears on both sides of a split. Only the first 20 s of each FaceForensics++ and Celeb-DF video are processed; UBFC videos are processed whole.

# 6. Evaluation protocol

No person on both sides. Model selection uses 5-fold GroupKFold inside the training split; groups are FF++ source pairs. A random split of windows would put windows of the same video in training and testing; the report measures how much that inflates the score.

Decisions frozen before testing. The model, the early-stopping epoch and the threshold are all chosen on training or validation data only.

Video-level scores are the headline; window-level scores are reported alongside.

Generalisation. The FF++-trained models are tested unchanged on Celeb-DF (a different dataset with higher-quality fakes) and on UBFC-rPPG's genuine webcam videos, where any 'fake' verdict is a false alarm.

Heart-rate ground truth for a window comes from the oximeter's PPG waveform analysed with exactly the same spectral method, as the dataset's authors recommend, so both sides have the same frequency resolution.

| Metric | Meaning |
| --- | --- |
| MAE / RMSE (bpm) | mean absolute / root-mean-square difference between video and oximeter heart rate |
| Pearson r | linear agreement of the two heart-rate series |
| Bland-Altman bias, limits | mean difference, and the range holding 95% of differences (bias +/- 1.96 SD) |
| Within 5 bpm | share of windows whose estimate is within 5 bpm of the oximeter |
| ROC AUC | probability a random fake scores above a random real video; 0.5 is chance, 1.0 perfect (95% CI by bootstrap) |
| Balanced accuracy | mean of the detection rate on fakes and on reals; robust to the 5:1 class imbalance |
| EER | error rate at the threshold where false alarms equal misses |
| Average precision | area under the precision-recall curve |
| False-alarm rate | share of genuine videos called fake |

Table 8. Metrics.

# 7. Results: heart rate

Not run yet. Heart-rate error against the pulse oximeter will appear here after you run:

python -m pulsecheck extract ubfc --root C:\datasets\UBFC-rPPG --max-seconds 0

python -m pulsecheck eval-hr

## 7.1 Compression

Not run yet. Heart-rate error at several H.264 compression levels will appear here after you run:

python -m pulsecheck compression-study --root C:\datasets\UBFC-rPPG --limit 20

# 8. Results: deepfake detection

Not run yet. Detector training and testing on FaceForensics++ will appear here after you run:

python -m pulsecheck extract ffpp --root C:\datasets\FaceForensics --compression c23

python -m pulsecheck extract celebdf --root C:\datasets\Celeb-DF-v2

python -m pulsecheck train --cross Celeb-DF=cache/celebdf --real UBFC=cache/ubfc

python -m pulsecheck train-cnn --cross Celeb-DF=cache/celebdf --real UBFC=cache/ubfc

# 9. Results: generalisation

Not run yet. Cross-dataset (Celeb-DF) and genuine-video (UBFC) results will appear here after you run:

see Chapter 8 commands (--cross and --real)

# 10. Web application and deployment

## 10.1 Backend

FastAPI (Python) serves the API and the static site from one process. Analysis runs on a single background thread, because a free Hugging Face CPU Space has 2 virtual CPUs; the browser polls for progress, which is reported frame by frame during landmarking. Uploads are streamed to a temporary file with a size cap (60 MB), analysed for at most 20 s, and deleted as soon as the job finishes; nothing is stored or logged.

| Endpoint | Purpose |
| --- | --- |
| POST /api/analyze | upload a video (multipart) and a method; returns a job id |
| GET /api/jobs/{id} | status, stage, progress, queue position, and the result when done |
| GET /api/info | version, loaded models and their validation/test AUC, limits |
| GET /api/reports | the evaluation reports (real datasets only) |
| GET /healthz | liveness probe used by Docker |
| GET /api/docs | interactive OpenAPI documentation |

Table 9. HTTP API.

## 10.2 Frontend

A single static page in plain HTML, CSS and ES-module JavaScript, with Plotly.js 3.7.0 (from the jsDelivr CDN) for interactive charts. Four views: Analyze (upload, 15 s webcam recording through MediaRecorder at 6 Mbit/s because compression destroys the pulse, every step of the analysis as charts, the patch overlay, the verdict), Results (the evaluation dashboards), How it works (a live in-browser lab for the Fourier maths) and About (model card).

Charts follow one set of rules: a colour palette validated for colour-vision deficiency in light and dark mode, 2 px lines, rounded bar ends, hairline grids, a legend whenever there are two or more series, and a 'Table' button on every chart that shows the same numbers as an accessible table. Each skin patch keeps the same colour in the overlay image and in every chart.

## 10.3 Deployment

A Dockerfile builds on python:3.14-slim, adds the shared libraries OpenCV and MediaPipe need (libgl1, libglib2.0-0, libportaudio2), installs the pinned requirements as a non-root user with uid 1000 (as Hugging Face requires), downloads and checksum-verifies the face model, and starts Uvicorn on port 7860 with a health check. The README front matter (sdk: docker, app_port: 7860) configures the Hugging Face Space, and the Hugging Face CLI uploads the project (it stores binary files with Xet automatically). Training always happens on the laptop; only the trained model files are uploaded.

# 11. How to reproduce

python -m venv .venv  &&  .venv\Scripts\activate

pip install -r requirements-dev.txt

pip install torch==2.14.1 --index-url https://download.pytorch.org/whl/cu126     # GPU training

python -m pulsecheck download-model

python -m pytest                                                            # all tests

python -m pulsecheck extract ubfc   --root C:\datasets\UBFC-rPPG --max-seconds 0

python -m pulsecheck eval-hr

python -m pulsecheck compression-study --root C:\datasets\UBFC-rPPG --limit 20

python -m pulsecheck extract ffpp   --root C:\datasets\FaceForensics --compression c23

python -m pulsecheck extract celebdf --root C:\datasets\Celeb-DF-v2

python -m pulsecheck train      --cross Celeb-DF=cache/celebdf --real UBFC=cache/ubfc

python -m pulsecheck train-cnn  --cross Celeb-DF=cache/celebdf --real UBFC=cache/ubfc

python -m pulsecheck report-doc --author "Your Name"

python -m pulsecheck serve                                                  # http://127.0.0.1:7860

# 12. Testing

The repository has 40 automated test functions (pytest) in 8 files. They check the code against answers known in advance: the article's worked example (300 samples, step 12, 72 bpm), heart-rate recovery at off-bin rates, filter behaviour, all three rPPG methods (including the lamp-flicker case), coherence and features, the dataset loaders on miniature folder trees laid out like the official downloads, the metrics against hand-computed values, the web API end to end, and the CNN on a learnable toy task.

About synthetic data. To test the pipeline end to end, the test suite renders short clips with a known pulse painted onto a public-domain photograph. These exist only to check that the code computes the right answer; they are never used for any reported result, model, or anything shown on the website.

# 13. Limitations and responsible use

Not a forensic tool. A verdict is a screening signal and must never be used to accuse anyone.

High-quality face swaps can inherit their source video's pulse (Seibold et al., 2025).

Compression, head movement, uneven or flickering light, low resolution and heavy make-up all weaken the signal.

Darker skin absorbs more light, which lowers the pulse's strength; UBFC-rPPG has few darker-skinned subjects, so accuracy there is not established by this work.

The detectors are trained on YouTube-style FaceForensics++ videos; webcam clips and new generators are different distributions, so performance on them is less certain than the test numbers.

Privacy: the website processes uploads in memory and deletes them; the datasets are never redistributed, and the UBFC restriction on subjects 21 and 27 is respected.

# 14. Technology stack

| Component | Version | Used for |
| --- | --- | --- |
| Python | 3.14.7 | language for the pipeline, training and server |
| NumPy | 2.5.4 | arrays and vectorised signal maths |
| SciPy | 1.18.1 | Butterworth filters, periodograms, Welch coherence, detrending, statistics |
| OpenCV | 5.0.0.93 | video decoding (FFmpeg backend), colour spaces, drawing |
| MediaPipe | 1.1.0 | Face Landmarker: 478 facial landmarks per frame |
| scikit-learn | 1.9.1 | classical classifiers, GroupKFold, metrics, permutation importance |
| PyTorch | not installed | the CNN, trained with CUDA on an NVIDIA RTX 3050 (4 GB) |
| joblib | 1.6.0 | saving the classical model |
| FastAPI | 0.143.0 | web API |
| Uvicorn | 0.54.0 | ASGI web server |
| python-multipart | 0.0.32 | video uploads |
| Plotly.js | 3.7.0 | interactive charts in the browser |
| pytest | 9.1.1 | tests |
| imageio-ffmpeg | 0.6.0 | H.264 encoding for the compression study and test clips |
| matplotlib | 3.11.2 | figures in this report |
| python-docx | 1.2.0 | this report |
| Docker | python:3.14-slim | container image |
| Hugging Face Spaces | Docker SDK | public hosting |

Table 10. Software used (versions read from the installed packages).

# 15. Glossary

| Term | Meaning |
| --- | --- |
| PPG / rPPG | photoplethysmography: measuring the pulse with light; remote PPG does it from video |
| ROI / patch | region of interest: an area of skin whose pixels are averaged |
| Band-pass filter | keeps only a range of frequencies (here where a heartbeat can be) |
| Fourier transform / FFT | splits a signal into the frequencies inside it |
| SNR | signal-to-noise ratio: power on the pulse versus power elsewhere |
| Coherence | how consistently two signals move together at a given frequency (0 to 1) |
| PPG map | an image whose rows are patches and columns are time, holding each patch's pulse |
| CNN | convolutional neural network |
| GroupKFold | cross-validation that keeps every group (person) within one fold |
| AUC | area under the ROC curve |
| ITA | individual typology angle: a skin-tone measure from CIELAB colour |

Table 11. Terms.

# 16. References

[1] W. Verkruysse, L. O. Svaasand, J. S. Nelson. Remote plethysmographic imaging using ambient light. Optics Express 16(26), 2008. https://doi.org/10.1364/OE.16.021434

[2] G. de Haan, V. Jeanne. Robust pulse rate from chrominance-based rPPG. IEEE Trans. Biomedical Engineering 60(10), 2013. https://doi.org/10.1109/TBME.2013.2266196

[3] W. Wang, A. C. den Brinker, S. Stuijk, G. de Haan. Algorithmic principles of remote PPG. IEEE Trans. Biomedical Engineering 64(7), 2017. https://doi.org/10.1109/TBME.2016.2609282

[4] U. A. Ciftci, I. Demir, L. Yin. FakeCatcher: Detection of synthetic portrait videos using biological signals. IEEE TPAMI, 2020. https://arxiv.org/abs/1901.02212

[5] S. Bobbia, R. Macwan, Y. Benezeth, A. Mansouri, J. Dubois. Unsupervised skin tissue segmentation for remote photoplethysmography. Pattern Recognition Letters 124, 2019. https://doi.org/10.1016/j.patrec.2017.10.017

[6] A. Rossler et al. FaceForensics++: Learning to detect manipulated facial images. ICCV, 2019. https://arxiv.org/abs/1901.08971

[7] Y. Li, X. Yang, P. Sun, H. Qi, S. Lyu. Celeb-DF: A large-scale challenging dataset for DeepFake forensics. CVPR, 2020. https://arxiv.org/abs/1909.12962

[8] C. Seibold, E. L. Wisotzky, A. Beckmann, B. Kossack, A. Hilsmann, P. Eisert. High-quality deepfakes have a heart! Frontiers in Imaging 4, 2025. https://doi.org/10.3389/fimag.2025.1504551

[9] X. Liu et al. rPPG-Toolbox: Deep remote PPG toolbox. NeurIPS Datasets and Benchmarks, 2023. https://arxiv.org/abs/2210.00716

# Appendix A. All detector features

| # | Name | Description |
| --- | --- | --- |
| 1 | snr_pooled | pooled SNR |
| 2 | entropy_pooled | spectral entropy |
| 3 | hr_spread_bpm | rate spread |
| 4 | hr_range_bpm | rate range |
| 5 | coh_mean | mean coherence |
| 6 | coh_min | min coherence |
| 7 | corr_mean | mean correlation |
| 8 | corr_min | min correlation |
| 9 | lag_mean_abs_s | mean lag |
| 10 | hf_ratio_pooled | pooled HF ratio |
| 11 | snr_forehead_left | SNR FhL |
| 12 | dhr_forehead_left | rate gap FhL |
| 13 | loo_corr_forehead_left | agreement FhL |
| 14 | hf_ratio_forehead_left | HF ratio FhL |
| 15 | rel_amp_forehead_left | rel. amplitude FhL |
| 16 | flicker_forehead_left | flicker FhL |
| 17 | snr_forehead_right | SNR FhR |
| 18 | dhr_forehead_right | rate gap FhR |
| 19 | loo_corr_forehead_right | agreement FhR |
| 20 | hf_ratio_forehead_right | HF ratio FhR |
| 21 | rel_amp_forehead_right | rel. amplitude FhR |
| 22 | flicker_forehead_right | flicker FhR |
| 23 | snr_cheek_left | SNR ChL |
| 24 | dhr_cheek_left | rate gap ChL |
| 25 | loo_corr_cheek_left | agreement ChL |
| 26 | hf_ratio_cheek_left | HF ratio ChL |
| 27 | rel_amp_cheek_left | rel. amplitude ChL |
| 28 | flicker_cheek_left | flicker ChL |
| 29 | snr_cheek_right | SNR ChR |
| 30 | dhr_cheek_right | rate gap ChR |
| 31 | loo_corr_cheek_right | agreement ChR |
| 32 | hf_ratio_cheek_right | HF ratio ChR |
| 33 | rel_amp_cheek_right | rel. amplitude ChR |
| 34 | flicker_cheek_right | flicker ChR |
| 35 | snr_nose | SNR Nose |
| 36 | dhr_nose | rate gap Nose |
| 37 | loo_corr_nose | agreement Nose |
| 38 | hf_ratio_nose | HF ratio Nose |
| 39 | rel_amp_nose | rel. amplitude Nose |
| 40 | flicker_nose | flicker Nose |
| 41 | snr_chin | SNR Chin |
| 42 | dhr_chin | rate gap Chin |
| 43 | loo_corr_chin | agreement Chin |
| 44 | hf_ratio_chin | HF ratio Chin |
| 45 | rel_amp_chin | rel. amplitude Chin |
| 46 | flicker_chin | flicker Chin |
| 47 | coh_forehead_left__forehead_right | coherence FhL-FhR |
| 48 | corr_forehead_left__forehead_right | correlation FhL-FhR |
| 49 | lag_forehead_left__forehead_right | lag FhL-FhR |
| 50 | coh_forehead_left__cheek_left | coherence FhL-ChL |
| 51 | corr_forehead_left__cheek_left | correlation FhL-ChL |
| 52 | lag_forehead_left__cheek_left | lag FhL-ChL |
| 53 | coh_forehead_left__cheek_right | coherence FhL-ChR |
| 54 | corr_forehead_left__cheek_right | correlation FhL-ChR |
| 55 | lag_forehead_left__cheek_right | lag FhL-ChR |
| 56 | coh_forehead_left__nose | coherence FhL-Nose |
| 57 | corr_forehead_left__nose | correlation FhL-Nose |
| 58 | lag_forehead_left__nose | lag FhL-Nose |
| 59 | coh_forehead_left__chin | coherence FhL-Chin |
| 60 | corr_forehead_left__chin | correlation FhL-Chin |
| 61 | lag_forehead_left__chin | lag FhL-Chin |
| 62 | coh_forehead_right__cheek_left | coherence FhR-ChL |
| 63 | corr_forehead_right__cheek_left | correlation FhR-ChL |
| 64 | lag_forehead_right__cheek_left | lag FhR-ChL |
| 65 | coh_forehead_right__cheek_right | coherence FhR-ChR |
| 66 | corr_forehead_right__cheek_right | correlation FhR-ChR |
| 67 | lag_forehead_right__cheek_right | lag FhR-ChR |
| 68 | coh_forehead_right__nose | coherence FhR-Nose |
| 69 | corr_forehead_right__nose | correlation FhR-Nose |
| 70 | lag_forehead_right__nose | lag FhR-Nose |
| 71 | coh_forehead_right__chin | coherence FhR-Chin |
| 72 | corr_forehead_right__chin | correlation FhR-Chin |
| 73 | lag_forehead_right__chin | lag FhR-Chin |
| 74 | coh_cheek_left__cheek_right | coherence ChL-ChR |
| 75 | corr_cheek_left__cheek_right | correlation ChL-ChR |
| 76 | lag_cheek_left__cheek_right | lag ChL-ChR |
| 77 | coh_cheek_left__nose | coherence ChL-Nose |
| 78 | corr_cheek_left__nose | correlation ChL-Nose |
| 79 | lag_cheek_left__nose | lag ChL-Nose |
| 80 | coh_cheek_left__chin | coherence ChL-Chin |
| 81 | corr_cheek_left__chin | correlation ChL-Chin |
| 82 | lag_cheek_left__chin | lag ChL-Chin |
| 83 | coh_cheek_right__nose | coherence ChR-Nose |
| 84 | corr_cheek_right__nose | correlation ChR-Nose |
| 85 | lag_cheek_right__nose | lag ChR-Nose |
| 86 | coh_cheek_right__chin | coherence ChR-Chin |
| 87 | corr_cheek_right__chin | correlation ChR-Chin |
| 88 | lag_cheek_right__chin | lag ChR-Chin |
| 89 | coh_nose__chin | coherence Nose-Chin |
| 90 | corr_nose__chin | correlation Nose-Chin |
| 91 | lag_nose__chin | lag Nose-Chin |

Table 12. The 91 features of one window.

# Appendix B. The 32-patch grid

Landmark numbers in row order: 104, 69, 108, 151, 337, 299, 333, 9, 116, 118, 101, 123, 50, 36, 187, 205, 6, 197, 195, 345, 347, 330, 352, 280, 266, 411, 425, 32, 208, 200, 428, 262.

# Appendix C. Command reference

| Command | What it does |
| --- | --- |
| download-model | fetch MediaPipe face_landmarker.task (checksum-verified) |
| fetch-splits | fetch the official FaceForensics++ train/val/test split lists |
| download-ubfc | fetch UBFC-rPPG DATASET_2 from Google Drive (needs gdown; resumable) |
| check-data | count a dataset download's videos against the official numbers |
| landmarks | draw the 468 landmark numbers and the six patches on a face |
| analyze | heart rate (+ verdict if a model is trained) for one video |
| synth-clip | render one synthetic clip with a known pulse |
| synth-dataset | render paired synthetic real/fake-like clips |
| extract | landmark a whole dataset into cached traces (slow, resumable) |
| eval-hr | heart-rate error against the oximeter (UBFC-rPPG) |
| compression-study | heart-rate error vs H.264 compression level (UBFC) |
| train | select, train and test the detector; write model + report |
| train-cnn | train the PPG-map CNN on the GPU (needs PyTorch); write model + report |
| report-doc | build the Word technical report from the code and reports/*.json |
| update-readme | write README.md's results table from reports/*.json |
| smoke-test | synthetic end-to-end run of every step (minutes, no datasets) |
| serve | run the website locally |

Table 13. python -m pulsecheck <command> --help shows every option.